# MediNet Packet Tracer Guide

Oct 26, 2026

How to open, run and present the MediNet hospital network lab (MediNet.pkt, Cisco Packet Tracer 9.0.1).

## 1. Open and check the lab

- Open Cisco Packet Tracer 8.x or 9.x, then File > Open > MediNet.pkt.

- Wait about 30 seconds until every link dot turns green. Switch ports pass through amber while spanning tree settles.

- Click any PC > Desktop > IP Configuration. It should show DHCP with an address ending in .11.

- Quick test: Doctor-PC > Desktop > Command Prompt > ping 192.168.50.10 gives four replies (the first may time out while ARP runs).

| Device in the file | Model | Address | VLAN |
| --- | --- | --- | --- |
| R1 | Cisco 2911 | 192.168.x.1 on G0/0.x | all (router-on-a-stick) |
| SW-Core | Cisco 2960-24TT | none | all |
| SW-Ground | Cisco 2960-24TT | none | 10, 20, 30 used |
| SW-ICU | Cisco 2960-24TT | none | 60 used |
| HMS-Server | Server-PT | 192.168.50.10 static | 50 |
| Admin-PC | PC-PT | 192.168.40.11 DHCP | 40 |
| Reception-PC | PC-PT | 192.168.10.11 DHCP | 10 |
| Doctor-PC | PC-PT | 192.168.20.11 DHCP | 20 |
| Lab-PC | PC-PT | 192.168.30.11 DHCP | 30 |
| Nurse-Station | PC-PT | 192.168.60.11 DHCP | 60 |
| ICU-Monitor | PC-PT | 192.168.60.12 DHCP | 60 |
| AP-Guest | AccessPoint-PT | SSID MediNet-Guest, WPA2-PSK | 99 |
| Guest-Phone | Smartphone-PT | 192.168.99.11 DHCP | 99 |

Cables: R1 G0/0 to SW-Core G0/1 (trunk); SW-Core Fa0/24 to SW-Ground Fa0/24 and SW-Core Fa0/23 to SW-ICU Fa0/24 (trunks); SW-Core Fa0/1 HMS-Server, Fa0/3 Admin-PC, Fa0/5 AP-Guest; SW-Ground Fa0/1 to Fa0/3 the three PCs; SW-ICU Fa0/1 Nurse-Station, Fa0/2 ICU-Monitor.

## 2. Device configurations

Paste each block into the device's CLI tab (Router> or Switch> prompt). They are already loaded in MediNet.pkt; use them to rebuild the lab or to explain it.

### R1

enable
configure terminal
hostname R1
no ip domain-lookup
banner motd # MediNet - authorised staff only #
interface GigabitEthernet0/0
 no shutdown
interface GigabitEthernet0/0.10
 description Reception
 encapsulation dot1Q 10
 ip address 192.168.10.1 255.255.255.0
interface GigabitEthernet0/0.20
 description Doctors-OPD
 encapsulation dot1Q 20
 ip address 192.168.20.1 255.255.255.0
interface GigabitEthernet0/0.30
 description Laboratory
 encapsulation dot1Q 30
 ip address 192.168.30.1 255.255.255.0
interface GigabitEthernet0/0.40
 description Admin-Records
 encapsulation dot1Q 40
 ip address 192.168.40.1 255.255.255.0
interface GigabitEthernet0/0.50
 description Servers
 encapsulation dot1Q 50
 ip address 192.168.50.1 255.255.255.0
interface GigabitEthernet0/0.60
 description ICU-IoT
 encapsulation dot1Q 60
 ip address 192.168.60.1 255.255.255.0
interface GigabitEthernet0/0.99
 description Guest-WiFi
 encapsulation dot1Q 99
 ip address 192.168.99.1 255.255.255.0
exit
ip dhcp excluded-address 192.168.10.1 192.168.10.10
ip dhcp excluded-address 192.168.20.1 192.168.20.10
ip dhcp excluded-address 192.168.30.1 192.168.30.10
ip dhcp excluded-address 192.168.40.1 192.168.40.10
ip dhcp excluded-address 192.168.60.1 192.168.60.10
ip dhcp excluded-address 192.168.99.1 192.168.99.10
ip dhcp pool RECEPTION
 network 192.168.10.0 255.255.255.0
 default-router 192.168.10.1
 dns-server 192.168.50.10
ip dhcp pool DOCTORS
 network 192.168.20.0 255.255.255.0
 default-router 192.168.20.1
 dns-server 192.168.50.10
ip dhcp pool LAB
 network 192.168.30.0 255.255.255.0
 default-router 192.168.30.1
 dns-server 192.168.50.10
ip dhcp pool ADMIN
 network 192.168.40.0 255.255.255.0
 default-router 192.168.40.1
 dns-server 192.168.50.10
ip dhcp pool ICU
 network 192.168.60.0 255.255.255.0
 default-router 192.168.60.1
 dns-server 192.168.50.10
ip dhcp pool GUEST
 network 192.168.99.0 255.255.255.0
 default-router 192.168.99.1
 dns-server 192.168.50.10
exit
ip access-list extended GUEST_IN
 permit udp any any eq 67
 permit udp 192.168.99.0 0.0.0.255 host 192.168.50.10 eq 53
 permit tcp 192.168.99.0 0.0.0.255 host 192.168.50.10 eq 80
 deny ip 192.168.99.0 0.0.0.255 192.168.0.0 0.0.255.255
 permit ip any any
ip access-list extended RECORDS_OUT
 permit tcp any any established
 permit icmp any any echo-reply
 permit ip 192.168.20.0 0.0.0.255 any
 permit ip 192.168.50.0 0.0.0.255 any
 deny ip any any
interface GigabitEthernet0/0.99
 ip access-group GUEST_IN in
interface GigabitEthernet0/0.40
 ip access-group RECORDS_OUT out
end
write memory

### SW-Core

enable
configure terminal
hostname SW-Core
vlan 10
 name RECEPTION
vlan 20
 name DOCTORS_OPD
vlan 30
 name LABORATORY
vlan 40
 name ADMIN_RECORDS
vlan 50
 name SERVERS
vlan 60
 name ICU_IOT
vlan 99
 name GUEST_WIFI
exit
interface GigabitEthernet0/1
 switchport mode trunk
interface range FastEthernet0/23 - 24
 switchport mode trunk
interface FastEthernet0/1
 switchport mode access
 switchport access vlan 50
interface FastEthernet0/3
 switchport mode access
 switchport access vlan 40
interface FastEthernet0/5
 switchport mode access
 switchport access vlan 99
end
write memory

### SW-Ground

Same seven vlan lines as SW-Core, then:

hostname SW-Ground
interface FastEthernet0/24
 switchport mode trunk
interface FastEthernet0/1
 switchport mode access
 switchport access vlan 10
interface FastEthernet0/2
 switchport mode access
 switchport access vlan 20
interface FastEthernet0/3
 switchport mode access
 switchport access vlan 30

### SW-ICU

Same seven vlan lines as SW-Core, then:

hostname SW-ICU
interface FastEthernet0/24
 switchport mode trunk
interface FastEthernet0/1
 switchport mode access
 switchport access vlan 60
interface FastEthernet0/2
 switchport mode access
 switchport access vlan 60

### Settings made in the GUI

| Device | Where | Setting |
| --- | --- | --- |
| HMS-Server | Desktop > IP Configuration | Static 192.168.50.10, mask 255.255.255.0, gateway 192.168.50.1, DNS 192.168.50.10 |
| HMS-Server | Services > DNS | DNS On; A record hms.medinet.local = 192.168.50.10 |
| HMS-Server | Services > HTTP | HTTP On; index.html replaced with the MediNet HMS portal page |
| AP-Guest | Config > Port 1 | SSID MediNet-Guest; WPA2-PSK; passphrase MediNet@2026 |
| Guest-Phone | Config > Wireless0 | SSID MediNet-Guest; WPA2-PSK; same passphrase; IP DHCP |
| All PCs | Desktop > IP Configuration | DHCP |

## 3. Live demo script

About 2 minutes per module. Each demo pairs a Packet Tracer action with the matching tab on the MediNet website.

| Module | Show in Packet Tracer | Say | Then on the website |
| --- | --- | --- | --- |
| Routing and DHCP | Doctor-PC > IP Configuration shows DHCP 192.168.20.11; Command Prompt ping 192.168.50.10 | One router port with seven subinterfaces routes between all departments and hands out every address. TTL 127 means one router hop. | DHCP tab: Release then Request on Doctor-PC to show DORA |
| Switching | R1 CLI show ip interface brief; SW-Core CLI show vlan brief and show interfaces trunk | Access ports carry one VLAN untagged; trunks carry all seven with 802.1Q tags. | Packet trace tab: Doctor opens HMS website, point at the tag ring on trunk hops |
| Server | Doctor-PC > Web Browser > hms.medinet.local | DNS on HMS-Server turns the name into 192.168.50.10, and HTTP serves the portal on TCP port 80. | Packet trace tab: Lab looks up hms.medinet.local |
| Wi-Fi and security | Guest-Phone browser hms.medinet.local loads; Guest-Phone ping 192.168.40.11 fails; Reception-PC ping 192.168.40.11 fails; Doctor-PC ping 192.168.40.11 works | Guests get only DNS and web. Records answer only doctors and servers. | Security tab: click a red cell to show the deciding ACL line; switch the ACLs off to compare |
| ICU and reliability | Nurse-Station ping 192.168.60.12: TTL 128, no router | ICU traffic stays on SW-ICU, so alarms survive a core failure. | ICU monitoring tab: Cardiac alarm; Link failures tab: cut SW-Core to SW-ICU, raise an alarm again |

Tip: switch Packet Tracer to Simulation mode (bottom right) and send a ping from Guest-Phone to 192.168.40.11. The envelope stops at R1 with a red cross: the ACL drop, visible.

## 4. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| PC shows 169.254.x.x | DHCP discover did not reach R1 | Check the PC's switch port VLAN (show vlan brief) and that the uplink is a trunk; press DHCP again in IP Configuration |
| First ping times out, the rest reply | ARP was resolving the next hop | Normal; ping again |
| All pings between departments fail | R1 G0/0 is down or a subinterface has the wrong dot1Q number | show ip interface brief on R1; G0/0 needs no shutdown |
| Guest-Phone has no address | SSID or passphrase differs from AP-Guest | Re-enter SSID MediNet-Guest and the WPA2 passphrase on both |
| Browser cannot find hms.medinet.local | DNS service off or wrong DNS server on the client | HMS-Server > Services > DNS On; client DNS must be 192.168.50.10 |
| Doctor-PC cannot reach Admin-PC | RECORDS_OUT edited or applied inbound | show access-lists and show running-config interface g0/0.40 |

Show commands worth knowing

- R1: show ip interface brief, show ip route, show ip dhcp binding, show access-lists

- Switches: show vlan brief, show interfaces trunk, show mac address-table

- PCs: ipconfig /all, ping, tracert 192.168.50.10